# Bode Plot & Simple Filter Design

Name: Mikal, Andrew

ID:

## Abstract:

A first-order low-pass filter with a voltage gain of 10 and a cutoff frequency of 3 kHz was designed and verified in LTspice using an LT1630 operational amplifier in a non-inverting configuration. Simulations included the frequency response, step response, and sinusoidal responses at 30 Hz, 3 kHz, and 300 kHz. The LT1368 op-amp was initially used but replaced due to signal attenuation. The LTspice results closely matched the theoretical transfer function, with minor deviations due to op-amp limitations.

## Design:

Figure 1: A first-order low-pass filter with a gain of 10 and a cutoff frequency of 3 kHz

H(s) = VoutVin = Av1 + sRC	(eq. 1)

Where:

Av = 1 + RfR2    (eq. 2)

H(s) = VoutVin = gain ×ω0ω0 + s	  (eq. 3)

ω0 = 2πf0

f0 = 12π × R1C1

ω0 = 2π × 12π × R1C1 = 1R1C1

Gain = 10

H(s) =10 ×1R1C11R1C1 + s   (eq. 4)

Cutoff frequency fc is defined as:

fc = 12πRC	(eq. 5)

For the design, the target parameters were Av = 10 and fc = 3 kHz. The values were chosen by rearranging equation (5). From this, RC = 5.03 × 10-5 s.

A standard capacitor value C1 = 10nF

R1 = 12π × fc × C1 = 12π(3 kHz)(10 nF)  ≃5.3 kΩ   (eq. 6)

A standard 5.3 kΩ resistor was selected.

To achieve the desired gain of 10, the ratio of the feedback resistor from equation (2).

Av = 1 + RfR2

Choosing R2 = 10 kΩ  &&  Rf = 90 kΩ

Using the transfer function from equation (4)

Y(s) = 1s × 10 ×1R1C11R1C1 + s

Now taking the inverse Laplace

y(t) = L-1 { 1s × 10 ×1R1C11R1C1 + s} = 10(1-e-1R1C1)u(t)

Where again, R1 × C1 =5.03 × 10-5 s

The LT1368 op-amp was tested initially, but the amplitude response exhibited attenuation due to internal compensation capacitance. The LT1630, with a higher gain-bandwidth product (GBW), was substituted and provided accurate performance near the designed cutoff.

## Verification:

Figure 2. AC Analysis (Bode Plot) using LT1368

Figure 3. AC Analysis (Bode Plot) using LT1630

Figure 4. Transient Analysis (Step Response) using LT1630, where → .tran 0 5m 0 10u                                     & → PULSE(0 0.5 0 1u 1u 1m 2m)

Figure 5. Sine Response using LT1630 of 30 Hz, where → Sine(0 1 30Hz)                                                      & → .tran 0 170m

Figure 6. Sine Response using LT1630 of 3 kHz, where → Sine(0 1 3 kHz)                                                      & → .tran 0 2m

Figure 6. Sine Response using LT1630 of 3 kHz, where → Sine(0 1 300 kHz)                                                      & → .tran 0 20u

### Sine-Wave Measurements (Transient)

Input amplitude: 1 V peak SINE(0 1 f)

Measured output: red trace (Vlp, op-amp output)

Gain in dB: 20 × log10(Vout_peak / Vin_peak)

Phase: φ = −360 × (ΔtT), where Δt is the delay between input and output zero crossings and T= 1f

| Frequency | Period (T) | Output Peak (V) | Gain (dB) | Phase (Theory) | Notes |
| --- | --- | --- | --- | --- | --- |
| 30 Hz | 33.3 ms | 10.0 V | +20.0 dB | ≈ - 0.6∘ | Correct gain |
| 3 kHz | 0.333 ms | 7.07 V | +17.0 dB | ≈ - 45∘ | Correct phase lag |
| 300 kHz | 3.33 μs | 0.3 V | -10.46 dB | ≈ - 89.4∘ | Strong attenuation |

## Discussion/Summary:

Initial tests with the LT1368 op-amp showed reduced output amplitude near and above the cutoff frequency, consistent with an insufficient gain-bandwidth product (GBW). Replacing it with the LT1630 restored the expected response.

Minor discrepancies between theory and simulation are due to finite op-amp GBW, input offsets, and parasitic capacitances.
 At 300 kHz (100× the cutoff), theory predicts −20 dB attenuation, while the measured output (0.3 V peak) corresponds to −10.46 dB—acceptable considering non-ideal effects and measurement precision.

The LT1630-based low-pass filter achieved the target 3 kHz cutoff and gain of 10, matching theoretical predictions and demonstrating expected phase behavior of first-order filters.

References

20251010_EE10L_LOWPASS_FILTER_AM.asc